Delay and Latency

Examples

- A link is 100km long and is transporting 100Byte packets.

  - At what bandwidth would propagation delay equal transmit delay?

  - Recalculate for 512Byte packets

(assume propagation velocity of 2 x 108 m/sec)

- A link is 2km long. Packets of 100Bytes are being propagated at 2 x 108 m/sec. Calculate B/W if Dprop = Dtrans

- A link has an RTT of 100ms and carries 1kB data packets. There is an overhead of 2xRTT before transmission of the first data packet. Calculate the time required to transfer a 1000KB file if the bandwidth is 1.5kbps.

- How wide is a bit on a 500Mbps link. What would its physical length be:

  - on copper wire where propagation velocity is 2.3x108m/s.

  - In free space where propagation velocity is 3x108m/s.

- A 128Kbps link is set up between earth and a station on Mars. The distance is 54x109 meters and propagation velocity is 3x108 m/s. Calculate:

  - The min RTT

  - Delay/Bandwidth product

  - How long would it take to transfer a 5MB picture file from Mars to Earth