| 2.4.2.4.1.1.13 | Low temperature test 2 | Assembly of parts as- installed in vehicle The retractors are to be stored for a period of 24 h at -40°C  2°C. The mechanical function test shall be carried out within one minute after removal from the conditioning cabinet . | 21 | V15 – V20 No functional impairment | 6 |  |  |  | Requirement : - No functional impairment | OK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.4.2.4.1.1.14.1 a | Blocking behaviour 2 ( in new condition) Vehicle-sensitive locking system (VSI- System Vehicle Sensitive Inertia) as per US-law after Environmental- simulation | 500mm residual webbing Acceleration ramp 17 g/s Testing forward pos. Testing backwards pos. Testing to spring side Testing to locking side | 22A | V01 – V06 Front side: 0,45 g – 0,45 g 14,7 mm – 21,3 mm Rear side: 0,45 g – 0,45 g 15,4 mm – 20,6 mm Spring side: 0,45 g – 0,45 g 13,8 mm – 21,2 mm Locking side: 0,45 g – 0,50 g 16,2 mm – 20,5 mm V15 – V20 Front side: 0,45 g – 0,45 g 15,4 mm – 20,0 mm Rear side: 0,45 g – 0,45 g 14,1 mm – 19,2 mm Spring side: 0,45 g – 0,45 g 14,9 mm – 20,4 mm Locking side: 0,45 g – 0,45 g 15,4 mm – 21,1 mm | 6 6 | 6 6 | 12/09 46/08 | 12/09 46/08 | Requirement: - Locking path ≤ 25,0mm | OK |
| 2.4.2.4.1.1.14.1 a | Blocking behaviour 2 ( in new condition) Vehicle-sensitive locking system (VSI- System Vehicle Sensitive Inertia) as per US- law after Environmental- simulation | 500mm residual webbing Looking acceleration 0,3g Acceleration ramp 6 g/s Testing forward pos. Testing backwards pos. Testing to spring side Testing to locking side | 22B | V01 – V06 No Lock V15 – V20 No Lock | 6 6 | 6 6 | 12/09 46/09 | 12/09 46/09 | Requirement: - Locking path ≤ 50,0mm | OK |
| 2.4.2.4.1.1.14.1 b | Blocking behaviour 2 ( in new condition) Vehicle-sensitive locking system (VSI- System Vehicle Sensitive Inertia) as per EG/ECE after Environmental- simulation | 300mm residual webbing Acceleration ramp 35 g/s Testing forward pos. Testing backwards pos. Testing to spring side Testing to locking side | 22C | V01 – V06 Front side: 0,35 g – 0,39 g 13,1 mm – 23,0 mm Rear side: 0,35 g – 0,35 g 15,8 mm – 23,0 mm Spring side: 0,35 g – 0,35 g 18,1 mm – 25,7 mm Locking side: 0,35 g – 0,37 g 13,8 mm – 20,4 mm V15 – V20 Front side: 0,35 g – 0,35 g 16,7 mm – 29,7 mm Rear side: 0,35 g – 0,35 g 14,0 mm – 20,4 mm Spring side: 0,35 g – 0,35 g 14,3 mm – 20,3 mm Locking side: 0,35 g – 0,35 g 14,6 mm – 21,2 mm | 6 6 | 6 6 | 12/09 46/08 | 12/09 46/08 | Requirement: - Locking acceleration 0,45g - Locking path  50mm | OK |
| 2.4.2.4.1.1.14.2 | Blocking behaviour (in new condition) Webbing sensitive locking system (WSI-System) after Environmental- simulation | 300mm residual webbing Acceleration ramp 90 g/s | 23A | V01 – V06 Lock at a =1,50 g – 1,80 g at extraction e = 23,4 mm – 39,7 mm V15 – V20 Lock at a =1,40 g – 1,60 g at extraction e = 25,7 mm – 43,8 mm | 6 6 | 6 6 | 12/09 46/09 | 12/09 46/09 | Requirement: - Locking acceleration 2,0g - Locking path  50mm | OK |
|  | Blocking behaviour (in new condition) Webbing sensitive locking system (WSI-System) after Environmental- simulation | 300mm residual webbing Acceleration ramp 35 g/s | 23B | V01 – V06 No Lock V15 – V20 No Lock | 6 6 | 6 6 | 12/09 46/09 | 12/09 46/09 | Requirement: - Locking acceleration 1,0g - Locking path  50mm | OK |
| 2.4.2.4.1.1.14.3 | Tilt angle after Environmental- simulation | 300mm residual webbing Angel of inclination 2°/s Testing forward pos. Testing backwards pos. Testing to spring side Testing to looking side | 24 | V01 – V06 Lock Front side: 18,7° - 23,0° Rear side: 20,8° - 23,9° Spring side: 18,5° - 20,6° Locking side: 18,2° - 21,6° Unlock Front side: 18,1° - 20,0° Rear side: 16,5° - 20,1° Spring side: 15,5° - 18,5° Locking side: 16,1° - 19,4° V15 – V20 Lock Front side: 20,0° - 21,8° Rear side: 21,1° - 22,0° Spring side: 19,1° - 20,2° Locking side: 19,6° - 21,1° Unlock Front side: 18,1° - 20,0° Rear side: 19,4° - 20,4° Spring side: 16,8° - 19,9° Locking side: 18,0° - 19,3° | 6 6 | 6 6 | 12/09 46/08 | 12/09 46/08 | Requirement: - Unlocked angle  15° - Locked angle  27° | OK |