Table of contents

Change log	3

1 Scope	6

2 Normative references	7

3 Terms, definitions and abbreviated terms	8

3.1	Terms from other standards	8

3.2	Terms specific to the present standard	10

3.3	Abbreviated terms	11

4 Principles	13

4.1	Overview of RHA process	13

4.2	Radiation effects on components	14

4.3	Evaluation of radiation effects	16

4.4	Phasing of RHA with the different phases of a space project	16

4.4.1	Phase 0: Mission analysis, Phase A: Feasibility	16

4.4.2	Phase B: Preliminary definition	16

4.4.3	Phase C: Detailed definition	16

4.4.4	Phase D: Qualification and production	16

4.5	Radiation reviews	17

5 Requirements	18

5.1	TID hardness assurance	18

5.2	TNID hardness assurance	21

5.3	SEE hardness assurance	24

Annex A (normative) Mission radiation environment specification – DRD	28

Annex B (normative) Radiation analysis report - DRD	30

Bibliography	32

Tables

Table 3-1: K values for P=0,9 and C=0,9 as function of the number of tested samples n	11

Table 5-1: EEE part families potentially sensitive to TID	18

Table 5-2: List of EEE part families potentially sensitive to TNID	21

Table 5-3: List of EEE part families potentially sensitive to SEE	24

Table 5-4: Worst case SET templates	25

Table 5-5: Environment to be assessed based on LETth	25

# Scope

This standard specifies the requirements for ensuring radiation hardness assurance (RHA) of space projects. These requirements form the basis for a RHA program that is required for all space projects in conformance to ECSS-Q-ST-60. RHA program is project specific. This standard addresses the three main radiation effects on electronic components: Total Ionizing Dose (TID), Displacement Damage or Total Non-Ionizing Dose (TNID), and Single event Effects (SEE).

Spacecraft charging effects are out of the scope of this standard.

In this standard the word “component” refers to Electrical, Electronic, and Electromechanical (EEE) components only. Other fundamental constituents of space hardware units and sub-systems such as solar cells, optical materials, adhesives, polymers, and any other material are not covered by this standard.

This standard may be tailored for the specific characteristic and constrains of a space project in conformance with ECSS-S-ST-00.

# Normative references

The following normative documents contain provisions which, through reference in this text, constitute provisions of this ECSS Standard. For dated references, subsequent amendments to, or revision of any of these publications do not apply. However, parties to agreements based on this ECSS Standard are encouraged to investigate the possibility of applying the more recent editions of the normative documents indicated below. For undated references, the latest edition of the publication referred to applies.

| ECSS-S-ST-00-01 | ECSS system - Glossary of terms |
| --- | --- |
| ECSS-Q-ST-10-09 | Space product assurance - Nonconformance control system |
| ECSS-Q-ST-30 | Space product assurance - Dependability |
| ECSS-Q-ST-30-11 | Space product assurance - Derating - EEE components |
| ECSS-Q-ST-60 | Space product assurance - Electrical, electronic, and electromechanical (EEE) components |
| ECSS-E-ST-10-04 | Space engineering - Space environment |
| ECSS-E-ST-10-12 | Space engineering - Methods for the calculation of radiation received and its erects, and a policy for design margins |
| ESCC 22900 | ESCC Basic Specification: Total dose steady state irradiation test method |
| ESCC 25100 | ESCC Basic Specification: Single Event Effect Test Method and Guidelines |
| MIL-STD-750E method 1080 (20 Nov. 2006) | Test methods for semiconductor devices - Single event burnout and single event gate rupture test |
| MIL-STD-750E method 1019 (20 Nov. 2006) | Test methods for semiconductor devices - Steady-state total dose irradiation procedure |
| MIL-STD-883G method 1019 (28 Feb. 2006) | Microcircuits - Ionizing radiation (total dose) test procedure |
| MIL-HDBK-814 (8 Feb. 1994) | Military Handbook: Ionizing dose and neutron hardness Assurance guidelines for microcircuits and semiconductor devices |

# Terms, definitions and abbreviated terms

## Terms from other standards

For the purpose of this Standard, the terms and definitions from ECSS-S-ST-00-01 apply, in particular for the following terms:

applicable document

approval

assurance

derating

EEE component

environment

equipment

failure

information

outage

recommendation

required function

requirement

review

risk

specification

standard

subsystem

system

test

traceability

validation

verification

For the purpose of this Standard, the terms and definitions from ECSS-Q-ST-60 apply, in particular for the following terms:

characterization

commercial component

screening

space qualified parts

For the purpose of this Standard, the terms and definitions from ECSS-E-ST-10-04 apply, in particular for the following terms:

dose

equivalent fluence

fluence

flux

linear energy transfer (let)

For the purpose of this Standard, the terms and definitions from ECSS-E-ST-10-12 apply, in particular for the following terms:

cross-section

displacement damage

LET threshold

multiple cell upset (MCU)

(total) non-ionizing dose, (T)NID, or non-ionizing energy loss (NIEL) dose

NIEL

projected range

radiation design margin (RDM)

sensitive volume (SV)

single event burnout (SEB)

single event dielectric rupture (SEDR)

single event effect (SEE)

single event functional interrupt (SEFI)

single event gate rupture (SEGR)

single event latch-up (SEL)

single event transient (SET)

single event upset (SEU)

solar energetic particle event (SEPE)

total ionizing dose (TID)

## Terms specific to the present standard

- component type TIDS

TID level at which the part exceeds its parametric/functional requirements

- component type TNIDS

TNID level at which the part exceeds its parametric/functional requirements

- enhanced low dose rate sensitivity (ELDRS)

increased electrical parameter degradation of a part when it is irradiated with a lower dose rate

- equivalent LET

averaged value of the LET curve inside a sensitive volume

- one sided tolerance limit

limit that will not be exceeded with a probability P and a confidence level C, assuming that TID degradation of electrical parameters follow a normal distribution law

- If <delta x> is the mean shift among tested population of n samples,  is the standard deviation of the shift, and K is the one sided tolerance limit factor, then:

- Delta XL = <delta x > + K σ, for increasing total dose shift

- Delta XL = <delta x > - K σ, for decreasing total dose shift

- K depends on the number of tested samples n, the probability of success P and the confidence limit C. K values are available in MIL-HDBK-814. A 3 sigma (K=3) approach is often used. With 10 samples tested it gives a probability of success P of 90% with a confidence level C of 99%. Table 3-1 gives the values of K as a function of the number of tested samples n for P=0,9 and C=0,9

Table 3-1: K values for P=0,9 and C=0,9 as function of the number of tested samples n

| n | K |
| --- | --- |
| 3 | 4,259 |
| 4 | 3,188 |
| 5 | 2,742 |
| 6 | 2,493 |
| 7 | 2,332 |
| 8 | 2,218 |
| 9 | 2,133 |
| 10 | 2,065 |

- radiation design margin (RDM)

ratio of TIDS over TIDL for TID and ratio of TNIDS over TNIDL for TNID

- radiation lot acceptance test (RADLAT)

see “radiation verification test”

- radiation verification test (RVT)

radiation test performed on sample coming from the same diffusion lot as the flight parts

- This test is also known as “radiation lot acceptance test (RADLAT)”.

- total ionizing dose level (TIDL)

calculated TID level received by the part at the end of the mission

- total non-ionizing dose level (TNIDL)

calculated TNID level received by the part at the end of the mission

## Abbreviated terms

For the purpose of this Standard, the abbreviated terms from ECSS-S-ST-00-01 and the following apply:

| Abbreviation | Meaning |
| --- | --- |
| APS | active pixel sensor |
| ASIC | application specific integrated circuit |
| CCD | charge coupled device |
| CDR | critical design review |
| DCL | declared part list |
| ELDRS | enhanced low dose rate sensitivity |
| EOL | end of lifetime |
| FMECA | failure mode effects and criticality analysis |
| GEO | geostationary Earth orbit |
| LET | linear energy transfer |
| MCU | multiple cell upset |
| MOS | metal oxide semiconductor |
| NCR | nonconformance report |
| NIEL | non-ionizing energy loss |
| PDR | preliminary design review |
| QR | qualification review |
| RADLAT | radiation lot acceptance test |
| RDM | radiation design margin |
| RHA | radiation hardness assurance |
| RVT | radiation verification test |
| SEB | single event burnout |
| SEDR | single event dielectric rupture |
| SEE | single event effect |
| SEFI | single event functional interrupt |
| SEGR | single event gate rupture |
| SEL | single event latch-up |
| SET | single event transient |
| SEU | single event upset |
| SRR | system requirement review |
| TID | total ionizing dose |
| TIDL | total ionizing dose level |
| TIDS | total ionizing dose sensitivity |
| TNIDL | total non-ionizing dose level |
| TNIDS | total non-ionizing dose sensitivity |
| TNID | total non-ionizing dose |
| TNIDL | total non-ionizing dose level |
| TNIDS | total non-ionizing dose sensitivity |
| WCA | worst case analysis |