Delay and Latency

Answers

Note 	when working with Bandwidth M means 1000 x 1000 = 1000000

when working with files M means 1024 x 1024 = 1048576

Tip	convert MB, KB and Bytes to bits and Km to metres

- A link is 100km (100000m) long and is transporting 100Byte (800b) packets.

  - At what bandwidth would propagation delay equal transmit delay?

Dprop = Distance / Speed

Dprop = 100000 / 2 x 108 = 0.0005s

=> Dtrans = 0.0005s

Dtrans = Length / BW

0.0005 = 800 / BW

BW = 800 / 0.0005 = 1600000bps

  - Recalculate for 512Byte packets

(assume propagation velocity of 2 x 108 m/sec)

512B = 4096b

0.0005 = 4096 / BW

BW = 4096 / 0.0005 = 8192000bps

- A link is 2km (2000m) long. Packets of 100Bytes (800b) are being propagated at 2 x 108 m/sec. Calculate B/W if Dprop = Dtrans

Dprop = Distance / Speed

2000 / 2 x 108  = 0.00001

0.00001 = 800 x BW

BW = 800 / 0.00001 = 80000000bps = 8Mbps

- A link has an RTT of 100ms and carries 1kB (1024b) data packets. There is an overhead of 2xRTT before transmission of the first data packet. Calculate the time required to transfer a 1000KB (1000 x 1024 = 1024000b) file if the bandwidth is 1.5kbps (1500bps).

Overhead = 2 x 100ms = 200ms

Dprop = 0.5 x RTT = 50ms

Dtrans per packet = 1KB / 1.5Kbps = 8 x 1024 / 1500 = 5.4613s

1000KB => 5.4613 x 1000 = 5461.3s

Total time = Overhead + Dprop + Dtrans = 200 + 50 + 5461.3 			= 5711.3s

- How wide is a bit on a 500Mbps (500000000bps) link. What would its physical length be:

Width = 1 / BW = 1 / 500Mbps

=> 1 / 5000000000 = 0.000000002s

  - On copper wire where propagation velocity is 2.3x108m/s.

2.3 x 108 x 0.000000002 = 0.46m

  - In free space where propagation velocity is 3x108m/s.

3 x 108 x 0.000000002 = 0.6m

- A 128Kbps (128000bps) link is set up between earth and a station on Mars. The distance is 54x109 meters and propagation velocity is 3x108 m/s. Calculate:

  - The min RTT

RTT = 2 x BW x Delay (2 x BW x  Dprop)

Delay = Distance / Speed (Dprop = Distance / Speed)

Dprop = 54x109  /  3x108 = 180s

min RTT = 2 x Dprop = 2 x 180 = 360s

  - Delay/Bandwidth product

note this is Delay times Bandwidth, NOT Delay divided by Bandwidth

BW x RTT =

128000 x 360 = 46080000bps

=> 46.08Mb => 5.76MB

  - How long would it take to transfer a 5MB picture file from Mars to Earth

Dtrans  = Length / BW =5 x 1024 x 1024 x 8 / 128000 = 327.68s

Time = Dprop + Dtrans  = 180 + 327.68 = 507.68s